Fiber-Optic IR Sticker

Jeff Morrison

Multimedia Device Control Systems

These systems provide end-users convenience in cooperatively controlling and organizing multimedia devices such as DVD Players/Recorders, DVR/PVRs (TiVos), Televisions, VCRs, Cable & Satellite receivers, audio receivers, etc. A multimedia device control system (MDC) works by interfacing with all of the devices and managing them individually from one, central hub. This allows for a reduction in the number of remote controls (as well as the number of remote control modes) and can also provide for automated coordination between the devices.

If a user’s home media setup includes a television, a separate audio receiver, and a separate satellite receiver, then the user must either leave some devices running or power each of them up individually. With a MDC, the user would only need to push a single button on the MDC’s remote which would initiate an automated sequence from the MDC to turn on all necessary devices.

MDC’s may also be useful in helping the end user to automate recordings. If the user’s setup is composed of an independent satellite receiver which passes through some type of recording device (say a DVD Recorder), setting up an automated scheduled TV recording can be complicated if even possible. In order for the automated scheduled recording to occur, the satellite tuner must be tuned to the proper channel at the time of airing of the show, and the DVD recorder must record only over the time period when the show is airing. With an MDC, these two completely independent devices can be controlled to do these necessary steps automatically. Using an MDC for this process also eliminates redundant programming (i.e. programming the receiver AND the dvd recorder to both do something over the same period of time).

How MDCs Could Control Independent Devices

There are currently several inter-device communications protocols available (specifically TCP/IP, IEEE1394 (FireWire), Infrared).

TCP/IP

TCP/IP is an extremely versatile and complex standard used, most commonly, with arbitrary computer systems. It is used in computer networking (including the Internet) almost without competition. It offers an extremely scalable architecture for communication on many levels of devices.

Unfortunately, with this broad spectrum of versatility comes complexity. TCP/IP has so many built in features for methods of communication and error correction that its implementation would be highly complicated and bloated. It would also require that each media device have a TCP/IP interface (for which almost none have now, and modification of older equipment to support this would certainly be far from cost effective).

IEEE 1394 (FireWire)

This protocol is very close to what is needed for MDC communication. In fact, it would be a good idea to include the ability to use FireWire with any type of MDC hub. FireWire offers high data rates for transferring data (such as video and audio) quickly. It also offers a method of command-based communication between devices. It is even already included in many digital video devices such as TiVos, digital video cameras, satellite receivers, and even some DVD Players and Televisions.

As good as this protocol sounds; it still retains one major flaw in that it is only compatible with devices that are already FireWire compatible. Old VCRs, LaserDisc players, BetaMax, etc would be left out of the possibilities for control by an MDC.

Infrared

Finally, we come to a solution that works for nearly any media device since the initial development of infrared (IR) remote controls in 1950 by the Zenith Electronics Corporation.

If there were a simple and unobtrusive way to allow for an MDC to communicate with devices using the IR technology that is already built in to them; no further equipment or equipment upgrades would be necessary to use an MDC with any media device setup. The IR interface from the MDC would simply make use of the standard method of device control. This would also allow for other types of controllers (i.e. remotes) to interface with the device without obstruction from the MDC’s interface.

One small problem with an IR interface for an MDC is line-of-site. In order for the MDC to communicate with a device, direct line-of-site would be constantly necessary. The solution to this problem is a simple translucent IR sticker that receives the IR signals through fiber-optic channels from the MDC, and refracts them towards the device’s IR receiver.

In order to prevent this sticker from interfering with other control devices (i.e. remote controls) it could be developed so that IR light can pass directly through it at any time.

Design & Operation of a Fiber-Optic IR Sticker

Using simple light geometry concepts, this sticker could be made from a plastic-injection mold, which would allow for quick and very low-cost manufacturing.

Fiber-Optic Concepts

The concept of a fiber optic cable is based on a property of light known as reflection. This property is defined by the idea that the angle a ray of light makes with a reflective surface (AKA the Angle Of Incident) will equal the angle the reflected ray of light makes with the reflective surface (AKA the Angle Of Reflection).

If a tubular transparent core is encased in a reflective material then regardless of the shape of the tube, the light that is presented at one end of the transparent core will do one of two things. It will either pass directly through the core without striking the walls of the tube, or it will strike the walls and be reflected back and forth continuously until it comes out the end opposite of entry. This allows light to be easily driven around angles or through long distances without worry of obstruction of line of site or discreet calculation of necessary points of reflection.

Light Refraction Concepts

In order for light to move between two points, it must travel through at least one transparent medium. When light moves from one medium to another, it may slow down slightly in speed resulting in a change in the angle of the light ray. This change in angle is consistent between any two given mediums.

Given that this change in angle is consistent, an index ratio can be described for the movement of light between the two given mediums. This value is known as the index of refraction for the given medium.

Using the known index of refraction for two mediums, the angle of the light as it passes from the first medium and through the second can be calculated with a formula known as Snell’s Law (figure 3.2.1):

Figure 3.2.1

N1 × Sin(θ1)

N2

N1 × Sin(θ1) = N2 × Sin(θ2)

- θ2 = Arcsin(            )

(where N1 is the index of refraction for medium 1,
and N2 is the index of refraction for medium 2)

The reason refraction is important to an IR sticker is that the grooves that are created across the surface of the IR sticker make an angled interface between the sticker’s material and air. The angle of the groove is carefully created so that the final angle of the ray of light coming out of the groove will shine on whatever the sticker is attached to. (See figure 3.2.2)

Figure 3.2.2

This creates a discreet planar projection of the light coming from the fiber-optic cable attached to the side of the sticker.

Right-Angle Discreet Planar Refraction

Here we demonstrate how several small areas of refraction can create a planar effect of light. This ensures that light is distributed across the media device’s receiver unit. (See figure 3.3)

Figure 3.3

Fiber-Sticker Interfacing

The interface between the sticker and the fiber-optic cable would need to be a sturdy one to prevent disconnection, but also to allow the fiber cable freedom of motion. This is easily implemented as a plastic jacket sheathing that is molded into the side of the sticker. This would assure a correct optical connectivity while still allowing some strict range of motion at the edge of the sticker. (See figure 4.1)

Figure 4.1